Sequential Extraction SOP

For soils with >10%C:

- User will have weighed 300mg* of soil into a 2mL glass Microsolv vial (Microsolv, Cat# 9502S-WCV) with a non-slit cap (Microsolv, Cat# 9502S-10C-B-M).

- 1mL of MilliQ H2O is pipetted into each sample and shaken on a thermomixer (Eppendorf, Cat# 2231000667) at RT and 2000 RPM for 2 hours.

- Centrifuge samples at 4500 RPM for 5 minutes on a plate spinner that accommodates 54 sample autosampler tray.

- Pull off supernatant using 200uL micropipette while avoiding disturbing and pulling up sediment and place in a pre-labeled 2mL Sorensen tube (part no.) [Make sure that most of the supernatant is taken off or else this will affect the rest of the extraction]. Supernatants are then placed in a sample box and stored at -80oC.

- 1mL of Optima LC/MS-grade methanol is pipetted into each sample and shaken on a thermomixer at RT and 2000 RPM for 2 hours.

- Centrifuge samples (Repeat Step 3)

- Pull of supernatant using 200uL micropipette (Step 4) but instead place supernatant into a pre-labeled 2mL Microsolv vial and capped with a non-slit cap. Supernatants are then placed in a sample box and stored at -80oC.

- 1mL of ethanol-stabilized chloroform is pipetted into the sample using a glass syringe or a pre-marked Pasteur pipette in a fume hood (IF PLASTIC MICROPIPETTE TIPS ARE USED, SAMPLES WILL BE CONTAMINATED WITH PEG AND UNUSABLE). Samples are then shaken on a thermomixer at RT and 2000 RPM for 2 hours.

- Centrifuge samples (Repeat Step 3)

- In a fume hood, pull off supernatant using 5-3/4” glass Pasteur pipette and transfer supernatant to 2mL glass Microsolv vial and cap with non-slip cap. Supernatants are then placed in a sample box and stored at -80oC.

For Soils and Sediment with <10%C:

- User will have weighed 2g* of soil into a 15mL Olympus tube (Genesee Sci., Cat# 28-103).

- 5mL of MilliQ H2O is pipetted into each sample and shaken on a vortex mixer (Thermo Scientific, Cat# 88880017TS) at RT and 1000 RPM for 2 hours.

- Centrifuge samples at 6000 RPM for 5 minutes in an angled rotor that accommodates 15mL falcon tubes.

- Pull off supernatant using 1mL micropipette while avoiding disturbing and pulling up sediment and place in a pre-labeled 15mL Olympus tube [Make sure that most of the supernatant is taken off or else this will affect the rest of the extraction]. Supernatants are then stored at -80oC with tubes resting on their sides to prevent cracking.

- Steps 2-4 are then repeated with the supernatant going into the same sample tube as before.

- 5mL of Optima LC/MS-grade Methanol is pipetted into each sample and shaken on a vortex mixer at RT and 1000 RPM for 2 hours.

- Centrifuge samples (Repeat Step 3).

- Pull of supernatant using 1mL micropipette the same as the H2O fraction. Supernatants are then stored at -80oC.

- 5mL of ethanol-stabilized chloroform is pipetted into the sample using a glass serological pipette in a fume hood (IF PLASTIC MICROPIPETTE TIPS ARE USED, SAMPLES WILL BE CONTAMINATED WITH PEG AND UNUSABLE). Samples are then shaken on a vortex mixer at RT and 1000 RPM for 2 hours.

- Centrifuge samples (Repeat Step 3).

- In a fume hood, pull off supernatant using 9” glass Pasteur pipette and transfer supernatant to 15mL Olympus tube. Supernatants are then stored at -80oC.

*Amount of soil can vary by how much sample the user has available as well as the type of soil e.g. peat will be less dense/fluffier and have more C than a permafrost sample and thus 300mg of soil would be more than necessary and would most likely not fit into a 2mL Microsolv vial.