Solid Phase Extraction (SPE) of DOM SOP

Bond Elut PPL SPE:

- Place labeled SPE cartridge (Agilent, Cat# 12105004) onto SPE vacuum manifold (Sigma-Aldrich (Supelco), Cat# 57250-U) with 15mL falcon tube underneath to collect waste.

- Prepare 5mL* of H2O sample in a 15mL Olympus tube (Genesee Sci., Cat# 28-103) by acidifying the sample to at least pH2 with 2µL of 37% HCl.

  - Depending on sample volume, sample may need to be diluted to 5mL with MilliQ H2O.

  - Depending on sample alkalinity and natural buffers within the sample, more 37% HCl may be needed to acidify the sample to pH2.

  - Test acidity with 0-6pH strip by pipetting 20µL of the sample on the pH strip.

- Activate each SPE cartridge with 3mL of Optima LC/MS-grade methanol and let excess drain off into waste.

- Equilibrate cartridge with 5mL 10mM HCl.

- Load the acidified sample onto the cartridge and allow the sample to flow through the column#.

- Wash the cartridge 17X+, # with 3mL of 10mM HCl, emptying waste tubes into a larger waste container once they are 80% full.

- Dry each cartridge with N2(g) until packing material returns to original color (about 5 minutes)

  - If top filter has moved during drying, tamp down gently with pipette tip.

- After cartridge is dry, expel any remaining HCl rinse in cartridge port and solvent guide needle by gently pushing N2(g) through the port.

- Replace cartridge onto original port and elute with 1.5mL of Optima LC/MS-grade MeOH into a 2mL Microsolv vial (Microsolv, Cat# 9502S-WCV) and cap with a non-slit cap (Microsolv, Cat# 9502S-10C-B-M) until ready to run. Sample is then placed in a sample box and stored at -80oC

*More than 5mL of sample can be used, especially if total volume of sample received is greater than 5mL and/or is of low concentration

#If samples are having a hard time flowing through with gravity and capillary action, gently pull vacuum to pull the sample or solvent through the column

+ Samples can be rinsed more than 17X depending on salt concentration