Modified Folch Extraction SOP

For Soils >10%C:

- User will have pre-weighed 300mg* of soil into a 15mL Olympus tube (Genesee Sci., Cat# 28-103)

- 1mL of MilliQ H2O will be added to the sample and put on a vortex mixer (Thermo Scientific, Cat# 88880017TS) at RT and 1000 RPM for 2 hours.

- Centrifuge sample at 6000 RPM for 5 minutes on an angled rotor that accommodates 15mL falcon tubes.

- Pull off supernatant using 200uL micropipette while avoiding disturbing and pulling up sediment and place in a pre-labeled 2mL Sorensen tube (part no.) [It is always made sure that most of the supernatant is taken off]. Supernatants are then placed in a sample box and stored at -80oC.

- 2mL of Optima LC/MS-grade MeOH is added to the sample using a glass serological pipette and vortexed for 5 seconds.

- 4mL of EtOH-stabilized CHCl3 is added to the sample in a fume hood using a glass serological pipette and vortexed for 5 seconds.

- 0.25mL of MilliQ H2O are added to the sample using a micropipette and sample is placed on a vortex mixer at RT and 1000 RPM for 1 hour.

- 1.25mL of MilliQ H2O are added to the sample and given a gentle shake by hand to get a bi-layer separation.

- Sample is then incubated overnight at 4oC

- Centrifuge sample at 6000 RPM for 5 minutes on an angled rotor that accommodates 15mL falcon tubes.

- In a fume hood, pull off bottom layer (lipid layer) using a 9” glass Pasteur pipette and split sample between a pre-lableled 2ml glass Mircosolv vial (Microsolv, Cat# 9502S-WCV) and a pre-labeled Waters vial (Waters, Cat# 186002805) and cap both with non-slit caps (Microsolv, Cat# 9502S-10C-B-M) (IF PLASTIC MICROPIPETTE TIPS ARE USED, SAMPLES WILL BE CONTAMINATED WITH PEG AND UNUSABLE). Samples are then placed in separate sample boxes and stored at -80oC

- Centrifuge sample again at 6000 RPM for 5 minutes on an angled rotor that accommodates 15mL falcon tubes.

- Pull off top layer (metabolite layer) using a 1mL micropipette or 9” glass Pasteur pipette and transfer supernatant into pre-labeled 2mL glass Microsolv vial (if using micropipette be careful not to plunge tip below bi-layer if there is remnant CHCl3). Sample is then placed in a sample box and stored at -80oC

Modified Folch Extraction SOP

For Soils and Sediments <10%C:

- User will have pre-weighed 2g* of soil into a 15mL Olympus tube+ (Genesee Sci., Cat# 28-103)

- 5mL of MilliQ H2O will be added to the sample and put on a vortex mixer (Thermo Scientific, Cat# 88880017TS) at RT and 1000 RPM for 2 hours.

- Centrifuge sample at 6000 RPM for 5 minutes on an angled rotor that accommodates 15mL falcon tubes.

- Pull off supernatant using 200uL micropipette while avoiding disturbing and pulling up sediment and place in a pre-labeled 15mL Olympus tube [It is always made sure that most of the supernatant is taken off]. Supernatants are then stored on their sides to prevent cracking at -80oC.

- 4mL of Optima LC/MS-grade MeOH is added to the sample using a glass serological pipette and vortexed for 5 seconds.

- 8mL of EtOH-stabilized CHCl3 is added to the sample in a fume hood using a glass serological pipette and vortexed for 5 seconds.

- 0.50mL of MilliQ H2O are added to the sample using a micropipette and sample is placed on a vortex mixer at RT and 1000 RPM for 1 hour.

- 2.50mL of MilliQ H2O are added to the sample and given a gentle shake by hand to get a bi-layer separation.

- Sample is then incubated overnight at 4oC

- Centrifuge sample at 6000 RPM for 5 minutes on an angled rotor that accommodates 15mL falcon tubes.

- In a fume hood, pull off bottom layer (lipid layer) using a 9” glass Pasteur pipette and transferred to a 15mL Olympus tube (IF PLASTIC MICROPIPETTE TIPS ARE USED, SAMPLES WILL BE CONTAMINATED WITH PEG AND UNUSABLE). Samples are then concentrated using N2 evaporator.

- Centrifuge sample again at 6000 RPM for 5 minutes on an angled rotor that accommodates 15mL falcon tubes.

- Pull off top layer (metabolite layer) using a 1mL micropipette or 9” glass Pasteur pipette and transfer supernatant into pre-labeled 2mL glass Microsolv vial (if using micropipette be careful not to plunge tip below bi-layer if there is remnant CHCl3). Sample is then placed in a sample box and stored at -80oC

*Amount of soil can vary by how much sample the user has available as well as the type of soil

+Depending on bulkiness of soil can also be done in 50mL tube